SmartSkale — IndiaLock Connect

TT ELOCK — Vendor Technical Questions  /  TT电子锁 — 厂家技术问题

Device / 设备: TT ELOCK, ID 82637294; firmware 79A-EN-V146_2625 / 78A-V5.1 7AVT1; Airtel 4G SIM (India).

Protocol / 协议: JT/T808 A13 ELOCK (2018-05-06). We connect to the lock directly over TCP (the protocol is TCP/socket, not MQTT).
我们按A13协议通过TCP直接与锁通讯（该协议为TCP/socket，非MQTT）。

Priority / 优先级: A = blocking / 阻塞   B = important / 重要   C = useful / 有用.   Please type answers in the right-hand column. / 请在右侧栏填写回复。

## 0.  About this sample lock  /  关于这台样机

| # | Question / 问题 | Prio | Vendor's Answer / 厂家回复 |
| --- | --- | --- | --- |
| S1 | Confirm the LockID (device number) of this sample. We read 82637294 on the label/screen — is that the LockID used in the protocol? 确认这台样机的锁号(LockID)。标签和屏幕显示82637294，这是协议中使用的锁号吗？ | B |  |
| S2 | The keypad password is 8 digits (00000000). The protocol Key field is 6 bytes (RF lock: 10 digits; password lock: 6 ASCII digits). Is this an RF lock or a password lock (Appendix 0), and how is this 8-digit password encoded into the 6-byte Key for remote commands? 键盘密码为8位(00000000)。协议Key字段为6字节(RF锁:10位;密码锁:6位ASCII)。按附录0，这是RF锁还是密码锁？远程指令时这8位密码如何编码进6字节Key？ | A |  |
| S3 | What is the SIM card ICCID inside the lock? (Needed to recognise the authentication code = ICCID + version.) 锁内SIM卡ICCID是多少？(协议规定鉴权码为ICCID+版本号，我们需要用它识别设备。) | A |  |
| S4 | Is this the 7B lock or the 4G lock model? (They use different base-station formats — 0xE1 vs 0x5D.) 这是7B锁还是4G锁型号？(两者基站信息格式不同——0xE1与0x5D。) | B |  |
| S5 | Confirm the exact firmware version, and what "79A-EN-V146_2625" and "78A-V5.1 7AVT1" each mean (main board vs communication module?). 确认确切固件版本，并说明"79A-EN-V146_2625"与"78A-V5.1 7AVT1"分别代表什么(主板与通讯模块?)。 | B |  |
| S6 | How are firmware updates applied to this device (OTA over the server? SMS? cable/tool?)? 如何为该设备进行固件升级(服务器OTA?短信?数据线/工具?)？ | C |  |
| S7 | Does the lock have any physical debug/serial ports for troubleshooting? 锁是否有用于排障的物理调试/串口？ | C |  |

## A.  Pointing the lock to our server  /  让锁连接到我们的服务器

| # | Question / 问题 | Prio | Vendor's Answer / 厂家回复 |
| --- | --- | --- | --- |
| A1 | Which server (IP or domain + TCP port) is lock 82637294 connected to right now? Is it a gps51-based platform? 锁82637294目前连接的服务器IP/域名和TCP端口是什么？是基于gps51的平台吗？ | A |  |
| A2 | How do we change the lock’s server IP/domain, TCP port and APN to our own server? Please give the exact method and command format for each: (a) SMS command (format + SMS password), (b) Bluetooth app / BLE command, (c) PC tool / cable, (d) command from your platform (e.g. 0x8103 params 0x0013/0x0018/0x0010, or 0x8105 cmd 2). 如何把锁的服务器IP/域名、TCP端口和APN改成我们自己的服务器？请提供每种方式的具体方法和指令格式：(a)短信(格式及短信密码)，(b)蓝牙App或蓝牙指令，(c)电脑工具/数据线，(d)平台下发(如0x8103参数0x0013/0x0018/0x0010，或0x8105命令字2)。 | A |  |
| A3 | Can you (the vendor) change the server address remotely for us? If yes, please first tell us the current values (so we can restore them), then set server and TCP port to values we send separately. 能否由贵司远程帮我们修改服务器地址？如可以，请先告知当前配置(以便恢复)，再设置为我们另行提供的服务器和端口。 | A |  |
| A4 | Does the Airtel SIM need a specific APN, or is it detected automatically? What is the current APN setting? Airtel SIM卡是否需要设置APN，还是自动识别？当前APN设置是什么？ | A |  |
| A5 | RECOVERY: if we seal from our platform and our remote unseal then fails, how is the lock opened? (keypad with the seal key? master/emergency password? BLE app? only by the vendor?) Safety prerequisite before our first remote seal. 应急：如果我们通过平台施封后远程解封失败，如何打开锁？(键盘输入施封密钥?万能/应急密码?蓝牙App?只能由厂家?)这是我们首次远程施封前的安全前提。 | A |  |
| A6 | After we send a lock/unlock command, what timeout should we expect before the lock acts and replies? 我们下发施封/解封指令后，锁执行并回复的超时时间大约是多少？ | B |  |

## B.  Protocol details missing from the A13 document  /  A13文档中缺失的协议细节

| # | Question / 问题 | Prio | Vendor's Answer / 厂家回复 |
| --- | --- | --- | --- |
| B1 | §10.1 business frame "数据长度": exactly which bytes does this length byte count? (from 0x2A to 0x23? including the 2-byte serial number? including the CRC?) §10.1业务帧"数据长度"字节具体统计哪些字节？(从0x2A到0x23?是否包含2字节业务流水号?是否包含CRC?) | A |  |
| B2 | Which CRC algorithm is the 1-byte CRC in the business frame, over which bytes? (e.g. CRC-8/MAXIM poly 0x31, XOR, or sum?) 业务帧1字节CRC用什么算法？计算哪些字节？(如CRC-8/MAXIM多项式0x31、异或、累加和?) | A |  |
| B3 | Please send one complete real hex example of (1) a Seal 0x8900 from the platform and (2) the lock’s 0x0900 operation reply (0x55), including business serial number and CRC (the document shows "$$"). 请提供一条完整真实的十六进制示例：(1)平台下发的施封指令0x8900，(2)锁回复的0x0900操作回复(0x55)，含业务流水号和CRC(文档中用"$$"代替)。 | A |  |
| B4 | The lock is currently locally sealed (by keypad). Can the platform unseal a locally sealed lock, and which key must be sent? 锁目前处于"本地施封"状态(键盘上锁)。平台能否解封本地施封的锁？应下发哪个密钥？ | A |  |
| B5 | Over BLE we saw a keypad unseal report LockStatus 0x60 ("unsealed") and result codes 0x70 then 0x90. Is 0x60 correct for a local/keypad unseal (Appendix 3 suggests 0xBx "local unsealed")? Why does a keypad unseal also report 0x70 "alarm released"? 我们通过蓝牙观察到键盘解封回报锁状态0x60(解封态)及结果码0x70、0x90。本地/键盘解封回报0x60是否正确(附录3为0xBx"本地解封态")？为何键盘解封还回报0x70"解除报警"？ | B |  |
| B6 | After an active upload 0x0900 (SubCmd 0x01–0x05), should the platform send the 0x61 "normal reply", or only the JT/T808 0x8001? (§10.3 shows both.) 锁主动上传0x0900(SubCmd 0x01–0x05)后，平台需回复0x61"普通回复"还是只回0x8001？(§10.3两者都写了) | B |  |
| B7 | What is the JT/T808 terminal ID (header BCD[6]) of this lock, and how does it relate to 82637294? 这把锁的JT808终端ID(消息头BCD[6])是多少？与82637294是什么关系？ | B |  |
| B8 | What authentication code will it send (0x0102)? ICCID + version? Does it need registration (0x0100) first? 锁会发送什么鉴权码(0x0102)？ICCID+版本号？是否需要先注册(0x0100)？ | B |  |
| B9 | Heartbeat interval, sleep behaviour, and how to keep the lock online (real-time mode) — which parameters? 心跳间隔、休眠机制，以及如何让锁保持实时在线——需设置哪些参数？ | B |  |
| B10 | Are all device times GMT+8? Can the clock be set to UTC or India time (GMT+5:30)? Is the 0x8103 parameter 0x002A time calibration required? 设备时间是否都是GMT+8？能否设为UTC或印度时间(GMT+5:30)？0x8103参数0x002A校时是否必须？ | B |  |
| B11 | Is A13 the correct protocol version for firmware 79A-EN-V146_2625 / 78A-V5.1? Is there a newer document? 固件对应的协议版本是A13吗？有没有更新的协议文档？ | B |  |
| B12 | Does the connection support TLS/encryption, or is it plain unencrypted TCP only? 连接是否支持TLS/加密，还是仅明文TCP？ | B |  |
| B13 | Are there any known differences between the A13 document and the real hardware for this model? 本型号是否存在A13文档与实际硬件之间已知的差异？ | B |  |

## C.  Vendor platform / API (gps51)  /  厂家平台与API

| # | Question / 问题 | Prio | Vendor's Answer / 厂家回复 |
| --- | --- | --- | --- |
| C1 | Does your platform support TT ELOCK seal/unseal (not the 锁车/断电断油 relay) via the API? Which cmdcode, which parameters (key?), and does the result include the lock’s operation result code (0x80/0x90/0x93...)? 贵司平台API是否支持TT电子锁施封/解封(非锁车/断电断油)？cmdcode是什么？参数(密钥)如何传？返回是否含锁的操作结果码(0x80/0x90/0x93等)？ | B |  |
| C2 | API domain, account/credentials, IP whitelist and pricing for our device(s)? API域名、账号、IP白名单及设备费用？ | C |  |
| C3 | Does "raw message forwarding (原始报文转发)" include the 0x0900 e-lock business frames? Format and transport? "原始报文转发"是否包含0x0900电子锁业务数据？格式和传输方式？ | C |  |
| C4 | Does the platform keep lock status (status bits 24–31) and TT alarms (0xE7/0xE8, rod cut, etc.)? 平台是否保存锁状态(状态位24–31)及TT扩展报警(0xE7/0xE8、锁杆剪断等)？ | C |  |
| C5 | Please provide the BLE protocol for this lock (service 0x0001, characteristics 0x0002/0x0003, 0xFF00/0xFF01). We want to configure and operate the lock over Bluetooth from our own interface. 请提供这把锁的蓝牙协议(服务0x0001，特征值0x0002/0x0003，0xFF00/0xFF01)。我们希望用自己的界面通过蓝牙配置和操作锁。 | B |  |
| C6 | Please provide the full SMS/text command list for this model (Appendix 5 lists only a few TT% commands). 请提供本型号完整的短信/文本指令列表(附录5只列了部分TT%指令)。 | B |  |

## D.  Certifications & support (for production)  /  认证与支持（面向量产）

| # | Question / 问题 | Prio | Vendor's Answer / 厂家回复 |
| --- | --- | --- | --- |
| D1 | Does the lock (or its cellular module) hold an India WPC-ETA approval, or an equivalent FCC/CE certification we can use to support an Indian filing? 锁(或其蜂窝模块)是否持有印度WPC-ETA认证，或可用于印度申报的等效FCC/CE认证？ | C |  |
| D2 | Does the lock/battery have BIS registration (India) or an equivalent safety certificate? 锁/电池是否有BIS(印度)注册或等效安全认证？ | C |  |
| D3 | Who is our direct engineering contact for connection issues, and how do we report a problem? 连接问题的技术对接人是谁？如何反馈问题？ | C |  |
| D4 | Firmware support duration for this model, and production lead time and pricing? 本型号固件支持年限，以及量产交期与报价？ | C |  |